유지 보수를 반영한 회분식 공정의 zero-wait 공정의 능동적 생산 계획

류준형, 이인범

포항공과 대학교 화학공학과

An Approach to Interactive Scheduling of Multiproduct Batch Process

under Zero-Wait Policy

Jun-Hyung Ryu and In-Beum Lee

Department of Chemical Engineering, Automatic Research Center, POSTECH

Introduction

Batch process has been the concerns of many tailor-made high value added low volume products. It can meet the change of business environment by supplying wanted products on time compared to continuous processes. That is to say, Batch process can effectively cope with the varying market changes. So Flexible price manipulation based on the market condition should be reflected to production planning. But researches of production planning and scheduling in batch processes could have not effectively followed that issue. The main direction of researches are based on the assumption that process condition is perfectly operational and take any orders neglecting the possible enormous process burden and try to consider the uncertainty in processing circumstances which is enhanced to on line scheduling between process and schedule.

So our research deals with the problem about the relation of customer and manufactures. The production scheduling takes a role between both parts and has its importance as the economic mediator. And we call this system as interactive scheduling system with three components, customer, scheduler and process operator.    And we will apply with zero wait process, which is proposed with new simple heuristic relation.

Interactive Scheduling

As you can see fig.1, instead of simple reckless production , we consider production information from customers about production species, unit cost and flexible due date. There are also customer index including customer's prospect, importance of product, chance for due date negotiation etc. If this criterion - decision index is violated, we cancel the order. On the other hand, from the plant, resource information and maintenance planning which tells when they can operate with reliable condition. For this, production scheduling considering maintenance is necessary. And production scheduler-Interactive Scheduling system (ISS) calculates from these data, selects the order and decide the production sequence. And we adjust with customers with its result for agreement of schedule and negotiation with regard to price. For the process, we transport the production plan for specific time with operating time and assignment of equipment.    The object of production scheduler, ISS is to maneuver between due dates and completion time of products for the maximum profit of operating process.

Fig.1 Diagram of interactive scheduling System

This process is progressed for the products scheduled to be produced in some time after. We can find time representation with Fig. 2.    This time relation is going through repeatedly.

Fig. 2 Time presentation of order progress under interactive scheduling

Maintenance

Kondili et al.[6] used discrete time representation for expressing time distribution considering various process condition, which contains maintenance issues. But their work has some disadvantages. New events takes place only the boundary of that time units, and this forces their mathematical expression to need massive variables for even tiny models and they did not consider the quality of maintenance.    On the other hand, Dedopoulos and shah [2] handled multipurpose process scheduling considering maintenance time. The past research about maintenance did not treat the quality of maintenance level, though the level should be set for various process situations. Maintenance problem is the issue which should be considered for the realistic process condition. That tells us the difference from the traditional scheduling algorithm by placing the restriction that time can be unavailable for certain period when repair and maintenance operation is needed for the harmonious production. Batch process is especially needed for its characteristic of frequent operation on-off. So our model consider maintenance with qualtiy of repair with three range expressed with requesting time period.

Zero-Wait policy

Many researches deals with zero wait process and we can refer to Jung et al [2]. The main difficulty of zero wait process is they have to decide starting time of products in addition to the sequence of products, which only matter in case of UIS,NIS. Jung et al[2] and other researches use head or tail time supplemented to processing time and tried to settle with MINLP,MILP. So we propose two step heuristic which only matters the sequence and the completion time is automatically determined once the sequence is set.

C’ij = Ci-1,j+tij    ( i>1)                                                                                                        (1)

OTi-1ij = Ci-1,j- C’ij-1        (OTi1=0,i>1, j>1)                                                                          (2)

MOTi-1,i,j = Max (OTi-1ij )    j= 1,.....,M                                                                            (3)

Ci,j=      Ci-1,j+tij + MOTi-1,i,j                                                                                                        (4)

Conclusion

The term, “ interactive scheduling ”, means relation of the process, scheduler and customer. Given process condition information and customer information, it decides the production schedule and operating time. Our main interest is to generate the feasible production schedule to make fully utilization of the process condition and to make greatest profits with that process by selecting the order. That needs the priority of production sequence should be given if some products have similar conflicting due dates. And if process condition of certain due date is not available, and if we can not possibly reorganize the sequence, it would be better not to produce that product based on some criterion which we decided as decision indexes. That attitude toward order reorganization is the contribution of this research.

Fig. 3 A Gantt Chart of Example schedule with proposed scheduling system.

Acknowledgement

This work was supported in part by the Korea Science and Engineering Foundation(KOSEF) through the Automation Research Center in POSTECH.

Reference

- I.T. Dedopoulos and N.Shah, "Long-Term Maintenance Policy Optimization in Multipurposte Process Plants", Trans IchemE, 74, 307-320, 1996

- Jung, J. H., Lee, H., Yang, D. R., Lee, I.-B.,"Completion times and optimal scheduling for serial multi-product processes with transfer and set-up times in zero-wait policy", Comput. Chem. Engng., 18, 537 ,1994

- E.Kondili,C.C; Pantelides and R.W.H.Sargent “A General Algorithm For Short-Term Scheduling Of Batch Operations-I.Computational Issues” Com.Che.Eng,17, No.2, pp211-227, 1993