- 비선형 분취용 액상 크로마토그래피의 구배용리

- 이재욱1, 정흥조1, 김학금1, 심왕근2, 문 희2

- 1서남대학교 환경화학공학부

- 2전남대학교 응용화학부

- Gradient Elution in Nonlinear Preparative Liquid Chromatography

- Jae Wook Lee, Heung Joe Jung, Hark Geum Kim, Wang Geun Shim, Hee Moon

Faculty of Environmental & Chemical Engineering, Seonam University

Faculty of Applied Chemistry, Chonnam National University

- Introduction

Gradient elution chromatography plays a very important role in separation of biomolecules in both analytical studies and large-scale processes. In most analytical application, linear elution chromatography is employed so that the migration of a component is not perturbed by the presence of other eluites. In preparative chromatography, however, the column is often overloaded in terms of feed volume or concentrations, or both. Therefore, interference effects become important since the migration molecules interfere with each other’s adsorption. As a result the adsorption isotherms are nonlinear and, under these conditions, the process is termed nonlinear, or overloaded, chromatography [Guiochon et al., 1994].

In elution chromatography, a pulse of a mixture is fed into the column and its components are eluted by a continuous stream of an eluent that binds less strongly to the stationary phase than any of the eluites. Elution is isocratic when the eluent strength is kept constant throughout the separation. In gradient elution, introduced almost five decades ago, the eluent strength is increased continuously during the chromatographic run by suitable manipulation of the mobile phase composition upon increasing or decreasing the concentration of an appropriate additive termed the mobile phase modulator. This facilitates the separation, in a single run, of all the components of the mixture, whether they are initially weakly or strongly bound to the stationary phase. For this reason, gradient elution is widely used for the separation of mixtures whose components display a broad range of retentivity. In gradient elution processes, concentration and composition of mobile phase in terms of the ionic strength, pH, organic solvent, temperature are changed with time[Lee et al., 1999]. Since the retention strengths of the feed components depend on the conditions of the modulator, the kinetic constants and distribution coefficients of these feed components are functions of mobile phase conditions. Generally, a mathmatical model is needed in order to carry out a complete engineering study of gradient-elution chrogmatography. However, models considered mass and kinetic transfer resistances, are quite complex. Thus, model equations have to solve by numerical techniques.

In this work, the effect of column overload in gradient elution is studied theoretically using a model based on a differential mass balance equation that is solved numerically using orthogonal collocation on finite element method. Elution profiles of a single component and the separation of binary mixtures having both constant and non-constant separation factors with changing modulator concentration are examined and contrasted with results obtained by isocratic elution under comparable conditions. Our ultimate goal is to development, scale-up, and optimization of gradient-elution chromatography for the separation of biomolecules.

- Theory & Model

### Adsoprtion Isotherm - In chromatography, the migration rate of the components of a mixture, and thus their separation, depends on how they distribute between the stationary and mobile phases. In overloaded chromatography, the concentration of an eluite, i in the stationry phase, qi, depends on its own local concentration, ci, as well as on the local concentrations of all the other eluites and of the mobile phase modulator, ϕ. Thus the adsorption isotherm, which formally describes the equilibrium concentrations of the components in the two phases, must account for the interdependence on the sorption behavior of the eluites and the modulator. In this work, Extended Langmuir equation and ideal adsorbed solutuion theory as a multicomponent adsorption isotherm are used. Extended Langmuir isotherm is given by

qi=aiexp(−Siϕ)Cpi1+j=1najλjexp(−Sjϕ)Cpj	for i=1,2,...,n			(1)

where, n means the modulator and λi is the ratio of Langmuir isotherm parameters,aibi.

### Governing Equations for Mobile and Stationary Phases - The following mathematical model takes into account the combined effects of axial dispersion, film mass-transfer resistance, and intraparticle diffusion.

−DL∂Cbi∂z2+υ∂Cbi∂z+∂Cbi∂t+(1−εb)εb∂qi(Cp1,Cp2,...,ϕ)∂t=0		(2)

(1−εp)∂qi(Cp1,Cp2,...,ϕ)∂t+εp∂Cpi∂t−εpDpi1r2∂∂rr2∂Cpi∂r=0	(3)

### Numerical Solution – The two PDEs, Eqs. (2) and (3) are discretized with the orthogonal collocation on finite element method, respectively. Then, these equations are converted to a first order ordinary differential equations. If NE elements, N interior collocation points, and NS  are used for the discretization of Eqs. (2) and (3), there will be NC(2NE+1)(2N+1) ODEs in the final ODE system. The ODE system is then solved with the LSODI subroutine from IMSL, which uses Gear’s stiff method for initial value ODE problems (Lee et al, 1997).

## RESULT& DISCUSSIONS

The simulation of a single component system was carried out with theoretic physical data listed in Table 1, To analyze the effect of nonlinear isotherm, simulations were made with three different concentration feed as shown in Figure 1. Because of the concavity of Langmuir isotherm, the resulting peak for a high feed concentration is not Gaussian, an effect of tailing was observed for high feed concentration. The first feed concentration (1.0 mol/m3), near to the linear range of isotherm, gives a nearly Gaussian peak, whereas the two other examples with greater sample concentrations show more asymmetry in their peaks. This phenomenon aries from the nonlinearity of the isotherm where usually carried out in prepartive scale chromatography.

Table 1. Parameters Values used for Simulation.

| Figure | Species | Pe | Conc [mol/m3] | Velocity [m/s] | Isotherm Parameters |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 300 | 1, 3, 5 | 3.0 x 10-3 | qm1=0.626 | b1=1.136 |
| 2 | 1 2 | 300 | 1.0 0.5, 1.0 | 3.0 x 10-3 | qm1=0.626 qm2=0.815 | b1=1.136 b2=3.556 |

Other parameters : NE=2, NF=5, NS=3, injection time=0.5 hr, L=0.1m

Figure 1. Influence of the injected sample size on concentration profile in nonlinear

chromatography

As illustrated in Figure 2, simulation was also carried out for a binary system with ideal adsorbed solution theory as a multicomponent isotherm. The feed stream composed of components A and B with equal concentratios is injected at the column entry. The simulation becomes more difficult with increasing biand high sample concentration. The choice of step-size and tolerence for integration plays an important role in the numerical procedure.

Under overloaded conditions, also, we have compared isocratic and gradient elution for single components and binary mixtures. The approaching method used in this study allows the investigation of the effect of gradient elution over a wide range of operating conditions and thermodynamic properties. It may not readily accessible in an experimental study. The insight gained on the separation of binary mixtures from this investigation can serve as a starting point for developing rational methods for the selection of conditions for the preparative separation of ternary and multicomponent mixtures.

Figure 2. Concentration profiles of binary system in nonlinear chromatography.

From the simulation results in the chromatography of a single component at high column load, we can observed that gradient elution as compared with isocratic elution exhibits distinctly improved characteristics by the mode of the modulator concentration. Our simulation results for binary system with constant separation factor over the modulator concentration range of interest indicate that under overloaded conditions gradient elution gives better results in terms of production rates and enrichment factors for both components of the mixture with yields comparable with those in isocratic elution. Also, simulations for the comparison of isocratic elution and gradient elution clearly demonstrate the advantage of gradient elution in terms of shortened separation time and increased peak heights with no loss of resolution. These results could be applied for the development, scale-up, and optimization of gradient-elution chromatography for the separation of biomolecules

## References

Guiochon, G., A.G., Shirazi, A.M., Katti, Fundamentals of Preparative and Nonlinear Chromatography, Academic(1994).

Lee, J.W. and C.H., Park, and H. Moon, “Adsorption and Desorption of Cephalosproin C on Nonionic Polymeric Sorbents”, Sep. Purification Technol, 12,1-11(1997).

Lee, J.W. and H. Moon, “Effect of pH on Adsorption of Cephalosporin C by a Nonionic Polymeric Sorbent”, Adsorption,  in print(1999).