막과 흡착을 이용한 자연유기물질의 제거

김성현, H. H. Ngo*, S. Vigneswaran*, A. Bidkar*, 문 희

NATURAL ORGANIC MATTER (NOM) REMOVAL USING

MEMBRANE-ADSORPTION HYBRID SYSTEM

S. H. Kim, H. H. Ngo*, S. Vigneswaran*, A. Bidkar* and H. Moon

Faculty of Applied Chemistry, Chonnam National University

*University of Technology, Sydney, Australia

# INTRODUCTION

The presence of natural organic matter (NOM) results in colour and major problems such as carcinogetic compounds formation upon chlorination (trihalomethanes - THMs), bacterial growth in the distribution system etc. The treatment may increase due to the increase of the amount of coagulants used caused by the NOM. The previous studies on water quality indicated that organics in significant quantities are present in surface water. Hence, the organics removal in water treatment becomes an important issue. In this regard, the new drinking water guidelines have imposed strict standards for dissolved organics in water. Basically, microfiltration (MF) cannot remove organics effectively itself. As such, the only way to make it be able to remove significant amount of NOM is to combine it with an adsorption unit (a microfiltration – adsorption hybrid system). Microfiltration – adsorption system would provide a techno-economical solution for the removal of NOM from water. The in-line powdered activated carbon  (PAC) addition is used in this hybrid system. The PAC is not only an effectively adsorption material but also it can lead to a decrease of surface fouling of membrane from concentration polarization and an increase of the fluid permeability of the cake formed on the membrane surface (Pirbazari et. al., 1992).

The purpose of this paper is to investigate the possible use of the microfiltration-adsorption system in removing the fulvic acid (one of major humic substances in water). A mathematical modeling is established for simulation this membrane hybrid system.

# EXPERIMENTAL

In this study, a series of membrane experiments were conducted with an incorporated in-line PAC addition (Figure 1). A spiral mixing device was made by winding PVC tube with diameter of 6 mm around a column 110 mm in diameter. The tube length and flow rate were changed based on the predetermined velocity gradient (G) and hydraulic residence time (mixing time). In the spiral mixing unit, the G-value was calculated using the formula which was established by Ngo et al. (1995). The feed solution was prepared by mixing Suwannee River fulvic acid in Milli-Q water. A wood based PAC with norminal size of 80% finer than 75 m was used. The PAC and solution were send together into a mixing device and went through to the CFMF cell. The CFMF cell has 9 filtration channels and the total membrane area is 3.24´10-3m2. The membranes used has a nominal pore sizes of 0.2mm. The pressure of the membrane was controlled at 20 kPA by two valves and pressure transducers at two points. The permeate quality was evaluated in terms of total organic carbon (TOC). The permeate flux was also measured periodically during the experiments. In order to study the kinetics of adsorption of fulvic acid by PAC, the experiments on adsorption equilibria and batch kinetics were also conducted.

Figure 1. Experimental set-up for microfiltration-adsorption hybrid system.

## RESULTS AND DISCUSSION

# The experimental and simulated values (from adsorption models of adsorption equilibrium and batch adsorption kinetics) are shown in F gures 2 and 3. Figure 2 indicates that PAC has high adsorption capacity for FA. Adsorption results were modelled using Langmuir,  Freundlich and Sips equations, which attempt to interprete a quantitative of the experimental data. Langmuir and Freunlich isotherm models showed the best fit with an error of 6.8 - 6.9% compared to Sips model (7.49%). Figure 3 shows the batch kinetic results and its prediction using the three different diffusion models. It was determined that the surface diffusion model (SDM) most accuracy fitted the experimental results.

Figures 4 and 5 showed that the FA removal efficiency increased with the increase of G-value and mixing time. The permeate flux was higher in the case of 4 min-mixing time compared to the other two cases (1.33 and 6.7 min-mixing time). The simulation results from SDM indicated that an increase in velocity gradient led to significant increase of kf and Ds. Thus, film mass-transfer coefficient and Ds are directly related to velocity gradient values. The kf and Ds were 9.6 x 10-4 and 9.3 x 10-13 respectively in all cases.

The results indicated that the average steady-permeate flux increased from 707.5 L/m2.h to 816.3 L/m2.h with the increase in the concentration of PAC from 100 mg/L to 260 mg/L.  The average flux was found to be lower when the concentration of PAC was increased to 500 mg/L and the decline rate in flux was up to 22% after 30 minutes of the experiment (Figure 6). Figure 7 shows the removal of FA increases with the increase of PAC concentration. The average removal of FA (as TOC) was 61, 71 and 89% respectively. The film mass-transfer coefficient kf and surface diffusion coefficient Ds were 0.0096 and 9.3 x 10-13   in all cases.

# CONCLUSIONS

From the above discussions, it can be concluded that the microfiltration-adsorption hybrid system can be successfully used in water treatment to remove the natural organic matter. The optimum conditions of mixing time, mixing intensity (G) and PAC amount are essential for a given water to be treated. The surface diffusion model was successful to predict the performance of this hybrid system

#### REFERENCES

1. Berninger, J. A., Whitley, R. D., Zhang, X., and Wang, N. H. L. (1991). A versatile model for simulation of reaction and nonequilibrium dynamics in multicomponent fixed bed adsorption processes, Comput. Chem. Eng., 15, 749.

2. Bourbigot, M.M., Cote, P. and Agbekodo, K. (1993). Nanofiltration: An Advanced Process for the Production of High Quality Drinking Water, Proceedings of the AWWA/IWSA Membrane Technology Conference, Baltimore, USA.

3. Fane, A.G and Vigeswaran, S (1993). Technical Report: Techno-economical evaluation of different membrane processes in drinking water treatment.

4. Moon, H. and Lee, W. K. (1983). Intraparticle diffusion in liquid-phase adsorption of phenols with activated carbon in finite batch adsorber, J. Colloids and Interface Science, 96, 1, 162-171.

5. Ngo, H. H., Vigneswaran, S. and Dharmappa, H. B. (1995). Optimization of direct filtration: experiments and mathematical models, J. Env. Tech., 16, 55-63.

6. Pirbazari, M., Badriyha, B. N., Ravindran, V. (1992). MF-PAC for treating waters contaminated with natural and synthetic organics, Journal of AWWA, Research and Technology, 95-103.

7. Vigneswaran, S, Ngo, H.H. and Visvanathan, C., Comparison of Membrane Hybrid System with Nanofiltration in Organic Removal, Asia-Pacific Conference on Sustainable Energy and Environmental Technology, Singapore, 64-71, 1996

Figure 2. PAC-FA equilibrium data with

model prediction

(using Freundlich isotherm) at 25oC

Figure 3. Batch adsorption kinetics of PAC-

FA with model prediction

(FA 8 mg/L, PAC 260 mg/L)

Figure 4. Performance of PAC-CFMF at different G-values. (Influent conc.=8 mg/L, PAC=260 mg/L, filtration time=30 min.)

Figure 5. Performance of PAC-CFMF at different mixing-time. (Influent conc.=8 mg/L, PAC=260 mg/L, filtration time=30 min.)

Figure 6. Effect of PAC doses on permeate flux (mixing time=4 min, G=160.4/s, influent FA=8mg/L, pressure =20kpa)

min.)

Figure 7. Performance of CFMF-PAC at different PAC doses (mixing time=4 min, G=160.4/s, influent FA=8mg/L, pressure =20kpa)