# 스티렌 모노머 반응기의 새로운 운전 조건 제안

임희진, 주은정, 이정석*, 장민호, 박선원

한국과학기술원, 화학공학과

*LG화학 SR연구소

# New Operation Strategy Development for A Styrene Monomer Reactor

Heejin Lim, Eunjung Joo, Jeongseok Lee*, Minho Chang, Sunwon Park

Dept. of Chem. Eng., KAIST

*SR Research Center, LG Chem.

## Introduction

Most of the industrial styrene plants carry out the adiabatic dehydrogenation reaction using the multiple reactors in series. Even though the adiabatic reactor needs more expensive equipment cost and operating cost, it has been applied to many companies because of their high conversion of ethylbenzene and the small usage of catalyst pellets.

There are many efforts to improve the productivity of styrene monomer production plants. But most of them are focused on improvement of the recovery section or the other equipments except the reactor and development of new dehydrogenation catalyst. The dehydrogenation reactor section is not considered because the experiment and the mathematical modeling of the radial-flow reactor are very difficult. The study about the reactor operating conditions is very important because the styrene monomer productivity is affected by the reactor.

In this study, a mathematical model for an adiabatic radial-flow styrene monomer reactor based on the chemical kinetics has been analyzed via computer simulation to exam important decision variables. Finally, new operating strategies have been proposed to maximize the productivity.

## System Description

The conversion of ethylbenzene and the selectivity of ethylbenzene to styrene are affected by reactor operating pressure, temperature, steam-to-hydrocarbon (oil) ratio (S/O ratio), and reactor load ([current ethylbenzene flow rate] / [design ethylbenzene flow rate]). The effects of each operating variable are shown in the following Table. 1.

| Table 1 Effects of operating variables |  |  |  |
| --- | --- | --- | --- |
| Variables | Characteristics | Increasing effects |  |
|  |  | Conversion | Selectivity |
| Operating pressure | • Constant in the current plant | ↓ | ↑ |
| S/O ratio | • Feed = EB + steam • Role of steam – A heating medium – A diluent – A coke suppresser | ↑ | ↓ |
| Temperature | • Endothermic reaction of EB dehydrogenation to SM | ↑ | ↓ |
| Load | • Effect on residence time | ↓ | ↑ |

## Sensitivity Analysis

A conversion of ethylbenzene and a production rate of styrene depend on the reaction temperature, pressure, S/O ratio, and load. In the current operation, temperature of each reactor, S/O ratio and load are decision variables because the reactor is operated under constant pressure. Since the inlet temperature is determined by S/O ratio, the effects of temperature are considered in two different aspects, S/O ratio change and Tmid change.

S/O ratio change

The S/O ratio depends on the conversion and the styrene monomer production rate because it has effects on residence time, inlet temperature, and styrene monomer partial pressure. The conversion and the production rate are known that they increase with S/O ratio increase. The reason of the increase is that the increase of steam ratio lowers the styrene monomer partial pressure and the reaction from ethylbenzene to styrene monomer goes to positive direction (EB → SM + H2). Increasing S/O ratio, the residence time is decreased because the total flow rate is increased. It can be predicted that the decrement of the conversion is followed by the increment of S/O ratio. Also, the increase of S/O ratio makes the inlet temperature higher because inlet steam has higher temperature than ethylbenzene stream. This causes the increment of the conversion and the production rate. The sensitivity analysis is based on the present operating point, 620℃ mid-point temperature and 110% load . In this case, the optimal S/O ratio is 1.2.

Tmid change

Since the dehydrogenation of ethylbenzene is an endothermic reaction, temperature is a very important factor for the reaction rate of ethylbenzene to styrene monomer. A mid-point naphtha heater between two reactors in series controls Tmid because the Tin depends on the S/O ratio. Increasing Tmid, the reaction rate is increased. The analysis is based on the present operating point, 110% load and 1.5 S/O ratios.

Load change

Load depends on the conversion and the styrene monomer production rate because it affects on residence time. The conversion is decreased but the production rate is increased with load increase. The reason of conversion increase is the decrease of residence time. Even though the total conversion of ethylbenzene is decreased with the residence time decrease, the effect of total flow rate increase overcomes this and the overall productivity is increased. The sensitivity analysis is based on the present operating point, 620℃ mid-point temperature and 1.5 S/O ratio.

The conversion and the styrene monomer production rate with 5% increment of decision variables are presented in Table 2. The comparisons with the sensitivities of decision variables are shown in the following Table 3.

| Table 2. Sensitivity analysis results on 5% increase of operating condition |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S/O ratio |  | Tmid (℃) |  | Load (%) |  |
| 5% increase | 1.63 | 1.71 | 620 | 651 | 110 | 115.5 |
| Conversion (%) | 68.09 | 68.40 (+0.46%) | 68.91 | 74.64 (+8.31%) | 68.91 | 67.62 (-1.87%) |
| SM mass flow (ton/day) | 463.81 | 466.30 (+0.54%) | 447.34 | 482.90 (+7.95%) | 447.34 | 461.19 (+3.10%) |

| Table 3. Comparisons between variables change |  |
| --- | --- |
| Effect | Comparison |
| EB conversion | Tmid > S/O ratio > Load |
| SM mass flow increase | Tmid > Load > S/O ratio |

From Tables 2 and 3, Tmid is the most sensitive variable on the conversion of ethylbenzene and the production of styrene monomer. To maximize the styrene monomer production rate of this plant, the recycle amount of ethylbenzene from downstream and the operating cost of the cooling and the separation systems should be considered.

## New Operation Strategy

In the previous section, considering only the styrene monomer production rate, maximizing Tmid can be the best policy. However, this can be the worst choice from the energy and productivity point of view. Currently, this reactor is operated maintaining Tin and Tmid at the same temperature. This operation brings fast deactivation of catalyst in reactor 1. To maximize the total productivity, it is necessary to find the optimum temperature control rule of Tin and Tmid.

Three different strategies changing Tmid are proposed in the following (Table 3). These are compared with the current operation policy in the cases of energy and productivity.

| Table. 3. Tmid changes in each strategy |  |
| --- | --- |
| Strategy | Tmid change (℃) |
| Constant ΔT | 605 ≤ Tmid ≤ 635 |
| Constant Tmid | Tmid = 620 |
| Increasing ΔT | 600 ≤ Tmid ≤ 640 |

Figure 1. Tin, Tmid and SM production rate versus DOS with the constant ΔT

Constant ΔT

In this policy, the temperature difference between Tmid and Tin is constant. Tin is varied from 600℃ to 630℃, and Tmid is changed 10℃ higher than Tin. In Figure 1, the styrene monomer production rate is decreased until DOS = 100. During this time, the catalyst deactivation effect on the production is stronger than the effect of temperature increase. After this point, the production rate grows because the deactivation rate is slowdown and the temperature-increasing rate is maintained.

Figure 2. Tin, Tmid and SM production rate versus DOS with the constant Tmid

Constant Tmid

In this strategy, Tmid is maintained constant at 630℃ during operation. Tin is also varied from 600℃ to 630℃. Because Tmid = 630℃ in early DOS is high enough to overcome the catalyst deactivation rate, this strategy makes the production rate increase continuously. In comparison with constant ΔT policy, this method shows 0.8% increment of energy in reactor 2 and 5.4% enhancement of total styrene monomer productivity. This strategy can react more amount of ethylbenzene than the current operation.

Increasing Tmid

In this policy, ΔT between Tin and Tmid is increased with time. Tin is varied from 600℃ to

Figure 3. Tin, Tmid and SM production rate versus DOS with increasing ΔT

630℃, and Tmid is increased from 600℃ to 640℃. This strategy shows similar profile of styrene production rate with constant ΔT policy.

In this section, three new different strategies are compared: constant ΔT, constant Tmid, and increasing ΔT. The first one is very similar to the current operating policy. The current operation maintains Tmid and Tin at the same temperature. Constant

Tmid policy shows the highest styrene monomer production rate but it deactivates the catalysts very fast. The last policy shows similar results with the current operation. Table 4 shows the results of the proposed methods. When the styrene monomer price is high and its production rate is lower than the consumption rate constant Tmid strategy is most efficient. But In the case of high energy cost, increasing ΔT policy is better.

| Table. 4. Strategy comparisons |  |  |
| --- | --- | --- |
| Strategy | Energy need for reactor 2 (%) | Total SM productivity (ton) |
| Current operation | 100 | 228792 |
| Constant ΔT | 101.6 | 253212 (+10.7%) |
| Constant Tmid | 102.4 | 266947 (+16.7%) |
| Increasing ΔT | 100.8 | 232249 (+1.5%) |

## Conclusion

The effects of the decision variables, S/O ratio, Tmid and load were investigated from the points of the styrene monomer productivity and the conversion. In this sensitivity analysis, the most effective variable on the productivity was Tmid.

To increase the catalyst run-length, new operating strategies were developed. These are different policies of Tmid operation. Among them, constant Tmid seems to be most effective from the energy consumption and the catalyst usage point of view.

## Acknowledgment

This work was partially supported by the Brain Korea 21 project.

## References

[1] H. Lim, E. Joo, J. Lee, M. Chang, and S. Park, “Mathematical modeling and simulation of styrene monomer reactor”, (2000)

[2] H. Lim, “Modeling and Operation Strategy Development for A Styrene Monomer Reactor”, M.S. Thesis, KAIST (2001)