- GAC 바이오필터에 대한 개념적인 모델링 개발

- 심왕근1, Chaudhary D. S2., Vigneswaran, S2, Ngo H.H.2,  문 희1

- 전남대학교 응용화학학부

University of Technology, Sydney

Development of a concenptual mathematical modelling for a GAC biofilter

- Wang Geun Shim1, Chaudhary D. S2., Vigneswaran, S.2 , Ngo H.H.2, Hee Moon1

1Faculty of Applied Chemistry, Chonnam National University, Kwangju 500-757

2Environmental Engineering Group, University of Technology, Sydney, P. O. Box 123, NSW 2007, Australia

### Introduction

Wastewater treatment and reuse is a sustainable solution in industries. Adsorption is very important treatment methods in removing the pollutants from water, and wastewater. Activated carbon is the most commonly used adsorption medium [1]. One of the major hurdles is the high cost of activated carbon. Further the regeneration is also expensive. Granular activated carbon (GAC) – biofilter can be one of the techno-economical systems used in wastewater treatment.

In water and wastewater treatment with activated carbon, biological activity on the carbon is the major one. This effect arises from the fact that pollutants present in wastewater are often both adsorbable and biodegradable. There are many previous studies on the biological activity on activated carbon in water and wastewater treatment. These studies indicated that biological growth onto activated carbon has advantages in organic and nutrients removal. Therefore, several processes have been developed, such as the biological activated carbon (BAC) systems in water and wastewater treatment. To meet the criteria of wastewater reuse for various purposes, a promising method is to use a filter bed, which has a dual function of adsorption and biodegradation. Biological adsorbers include the growth of a biofilm on adsorbent pellet surface for degrading the adsorbed contaminants.

During the last few decades, extensive studies have been conducted to develop mathematical models that describe the process dynamics of the BAC system. These models incorporate various simplifying assumptions for mass transport resistances, biofilm growth, biofilm biodegradation, and adsorption on activated carbon to explain the systems [1~5].

This paper aims at developing a mathematical model of a fixed bed GAC biofilter. The developed model in this study is based on the fundamental framework that assesses adsorption and biological degradation behavior of wastewater pollutants in fixed bed GAC systems. The model simulation results shown to illustrate the sensitivity of process dynamics to various kinetic model parameters.

- Theoretical Model

A simple mathematical model was developed to evaluate the organic removal in the BAC system. The systematic representation of the mass balance in a biological activated carbon system and the biofilm on the activated carbon are given in Figure 1.

The material balance for the substrate in a plug flow reactor can be represented by the advection-diffusion equation, which also incorporates the adsorption and the bio-reaction terms.

Figure 1 Schematic representation of the biofilm on an adsorbent surface.

Biofilm

Adsorbent

Liquid

qS

q

cb

cS

Diffusion

SS

∂C∂t=Dax⋅∂2C∂z2−u⋅∂C∂z−γBIO−γADS	 (1)

The rates of removal of the substrate from the liquid phase by adsorption and biodegradation are as follows:

γBIO=kmax⋅CKS+C⋅XS ,         γADS=1−εb⋅3N4⋅π⋅aP3 	 (2)

The biomass suspended in the bulk liquid can be modeled as follows:

∂XSusp∂t=Y⋅kmax⋅S⋅XSuspK+S+af⋅Xf⋅σεb⋅Lf−b⋅XSusp−XSusp⋅βθ⋅εb	(3)

In the modeling:

The substrate transport from the liquid phase to the adsorbent phase is represented using the linear driving force approximation (LDFA).

ρS⋅∂q∂t=ρS⋅kS⋅qS−q	 (4)

Biofilm growth and decay are incorporated the cell growth rate, which is the sum of the cell production rate due to degradation and its decay rate.

dLfdt=Y⋅kmax⋅SKS+S−btot⋅dr 	(5)

Biofilm diffusion and biodegradation are also included. Here, the following simplifications were used: (i) the curvature effect of the pellet activated carbon surface may be ignored, and (ii) the Glueckauf approximation will be used to describe intrapellet diffusion, diffusion of substrate through the film.

∂S∂t=Df⋅∂2S∂x2−Xf⋅kmax⋅SKS+S 	(6)

The change in bed porosity and specific surface area as a removal of biofilm growth were included.

1−εb1−εbo=fLf,    af=fLf	(7)

A suitable adsorption isotherm was used to describe the adsorption of the substrate onto the adsorbent. Several isotherms are available in the literature [1]. In this study, Langmuir equation was used to analyse the system.

- Results and Discussion

Dynamic mathematical model equations presented in this study were solved for phenol biodegradation. The kinetic parameters obtained from the past experimental  data were used as summarized in Table 1 [5]. The preferred means of numerically solving this complicated set of partial differential equations is to use the orthogonal collocation method (OCM), which can discretize the equations. The partial differential equations were reduced to a set of ordinary differential equations (ODEs) by this technique. The resulting sets of ODEs were solved using the subroutine DVODE that especially suitable for stiff systems of first order ODEs.

The model used for simulating the dynamics of this system depends on several parameters. Figure 2 shows the comparison of effluent concentrations for bed porosity and specific surface area. In the case of variable systems, faster breakthrough was observed. Here, the bed porosity and specific area changed with biofilm length and time. The effect of biofilm thickness on effluent concentration curves for model is shown in Figure 3. The initial biofilm thickness increased with (i) the decrease of breakthrough time, and (ii) the increase of biological effect of the system. Figure 4 shows the effect of biomass yield coefficient on the effluent concentration curves of fixed bed GAC system. These results indicated that a higher yield coefficient could increase the biological effects without decreasing the adsorption effects. At higher yield coefficients within a certain range, biological effects gradually decreased due to the change of bed porosity and specific surface area. When the bed porosity and specific area are independent of the biofilm length, the biological effect increased continuously.

Table 1. Kinetic parameters for model sensibility

| Parameter | Units | Values | Parameter | Units | Values |
| --- | --- | --- | --- | --- | --- |
| C | mg/L | 3.0 | Df | m2/s | 6.8E-10 |
| kmax | s-1 | 1.5E-4 | XSo | mg/L | 1.0E-7 |
| Ks | mg/L | 0.24 | Lfo | m | 1.0E-6 |
| Y | mg/mg | 0.34 | ρS | g/cm3 | 0.690 |
| Xf | mg/L | 6.4E+3 | εbo | - | 0.40 |

References

[1] Tien, C. "Adsorption Calculations and Modeling", Butterworth-Heinemann, Singapore (1994).

[2] Hozalski, R. M.and Bouwer, E. J., "Non-Steady State Simulation of BOM Removal in Drinking

Water Biofilters: Model Development", Wat. Res., 35, 198(2001).

[3] Leitao, A. and Rodrigues, A., "Dynamic Behavior of a Fixed-Bed Biofilm Reactor: Analysis of

the Role of the Intraparticle Convective Flow under Biofilm Growth", Biochem Eng J., 2, 1(1998).

[4] Speitel, G. E., Jr., Dovantzis, K. and DiGiano, F.A., "Mathematical Modeling of Bioregeneration

in GAC Columns", J. Environ. Eng., 113, 32(1987).

[5] Rittmann, B. E. and Chang, H. T., "Verification of the Model of Biofilm on Activated Carbon",

Environ. Sci. Technol. 21, 273(1987).

Figure 2 Comparison of effluent concentration for

bed porosity and specific surface area at

constant and variable values.

Figure 3 Effect of initial biofilm thickness on the

effluent concentration curves.

Figure 4 Effect of biomass yield coefficient on the effluent concentration curves.