액체크로마토그래피를 이용한 Cephalosporin C의 분리에 관한 연구

최도영,전종익,이재욱,정흥조

서남대학교 화학공학과

Separation of Cephalosporin C by a Nonlinear Preparative Liquid Chromatography

Do Young Choi , Jong Ik Jun , Jae Wook Lee , Heung Joe Jung

Depat. of Chem. Eng., Seonam Univ., Namwon 590-711

# Introduction

Antibiotics can be generally classified such as β-lactam, aminoglycoside, macrolide, peptide, polyether, etc., acdording to the physical and chemical properties. Nowadays, β-lactam has occupied about 70% in the worldwide antibiotic market. Among β-lactam antibiotics, especially, the demand for cephems has increased rapidly because of great stability in acid and activity rather than penems. Cephalosporin C, one of the cephems, produced by Cephalosporium acremonium has been widely used as an important starting material to make medical and pharmaceutical products.

The nomenclature for cephalosporin C is 3-acetoxymethyl-7-)D-5-amino-carboxybaleramido)-8-oxo-5-thia-1-azabicyclo[4,2,0]oct-2-ene-2-carboxylic acid, and generally called cephalosporin C. This is the hydrophilic and molecular weight is 415. The characteristic of β-lactam ring is a IR band of carboxy radical.

There are two big problems for the separation of cephalosporin C from the broths. First, the hydrophilic nature of the α-aminoadipyl side-chain of the cephalosporins renders them highly soluble in water and precludes direct solvent extraction methods. Hydrophobic compounds, such as penicillin G and penicillin V, can be recovered from the cultured broth efficiently and relatively cheaply by extraction at low pH with an organic solvent. Second, penicillin N is always existed in the broths with cephalosporin C and cephamycin. Therefore, it is necessary to remove penicillin N from its fermentation broths. Cephalosporin C is separated from penicillin N in special conditions, which the hydroxylation of β-lactam ring of penicillin N is faster and easier than cephalosporin C, and penicillin is unstable in acid rather than cephalosporin C.

Four techniques used either separately or sometimes in combination have been used for the isolation of cephalosporin from broths. These can be briefly classified as adsorption techniques, ion exchange techniques, derivatization followed by solvent extraction, and enzymic modification.

Adsorption-based separation technique is one of the promising methods for the separation of cephalosporin C from admixtures since it is a nondenaturing, highly selective, energy efficient, and relatively inexpensive process. A large-scale cyclic operation, which consists of three steps such as adsorption, desorption, and washing, has been generally applied so far.

This study is seperation of cephalosporin C nonlinear preparative liquid chromatography. In order to demonstrate the applicability of the model under different operative conditions, experiments were conducted using different input CPC concentration, mobile-phase concentration, injection volume and flow-rat

# Experimental

Cephalosporin C (CPC) titers were determined employing high-performance liquid chromatography(HPLC).

Table 1 Physical properties of polymeric sorbent

| Nonionic polymeric sorbents were used in this study. They are commercially available macroreticular and spherical resin particles of sulfonated polystyrene cross-linked with DVB(divinylbenzene) under the trade name SP207. These selections were based on the hydrophilic mature of the α-aminoadipyl side chain of the cephalosporin C as mentioned before. The physical properties of these sorbents are summarized in Table 1. The water content of fully swollen resin particles was determined from the weight loss of fully saturated samples after their drying in a vacuum oven at 80℃ for 48h, The arithmetic average particle diameter was determined by sorting a sample of wet resin particles with the aid of an optical microscope. Adsorbent | SP207 | Unit |
| --- | --- | --- |
| Particle size | 412 | μm |
| Particle density | 611 | m3kg-1 |
| Packing density | 365 | m3kg-1 |
| Particle porosity | 0.48 | - |
| Packing porosity | 0.40 | - |
| Moisture content | 50 | % |
| Surface area | 672 | m2g-1 |
| Average pore diameter | 6.02 | Nm |

We used a zinc-form cephalosporin C, which was obtained from Sigma (USA), as a standard material, and the experiments were carried out with a sodium-form cephalosporin C supplied by Cheil Food and Chemicals Inc. (Korea). Both forms of cephalosporin C were used as received from the manufacturers without further purification. All chemicals used in the experimental work were analytical reagent grade. Distilled and deionized water was used to prepare the test solutions.

Prior to a given experiment, sorbent particles were leached with Ethanol(100%) for 24h to ensure the wetting of the particles internal pores.

Experiment conditions

| HPLC | Vintage 2000 Orom tech |
| --- | --- |
| Mobile-phase | EtOH : Distilled water = 15:85 |
| Flow-rate | 1ml/min |
| Detector | UV-200 |
| Range | 260nm |
| Column | Acryl column. |
| ID | 2cm |
| Length | 16cm |

Apparatus

# Results and Discussion

Effect of CPC concentration		Effect of mobile-phase concentration

E

E

Effect of Injection volume		Effect of flow rate

# References

[1] Lee, J.W., Park, H.C., Moon, H. (1997). Adosorption and desorption of cephalosporin C on nonionic polymeric sorbents. Sep. and Purif. Tech. 12, 1.

[2] Lee, J.W., Jung, H.J., Moon, H. (1998). Effect of operating conditions of cephalosporin C on nonionic polymeric sorbents. Korean J. of Chem. Eng.,